Wireless Multimedia System

Professor Hsiao-kuang Wu (hsiao@csie.ncu.edu.tw)

Homework 4: (Due Nov 20, 2002)

1. In the last decades, progress in microelectronics and VLSI technology has fostered the widespread use of computing and communication applications in portable electronic devices. A new universal radio interface called Bluetooth has been developed enabling electronic devices to connect and communicate via short range radio connections. What particular group of companies proposed the Bluetooth technology? How does Bluetooth radio system coexist with cellular system and Wireless LAN system? Why dynamic slot allocation scheme could reach better performance? What is the slot time for Bluetooth system? What is the maximum user rate for Bluetooth? Does CDMA scheme be applied in one particular piconet? Does CDMA scheme be applied in one scatternet?

2. (A)  If a total of 33 MHz of bandwidth is allocated total particular FDD cellular telephone system which uses two 25 kHz simplex channels to provide full duplex voice and control channels, compute the number of channels available per cell if a system uses (1) 3-cell reuse (2) 7-cell reuse. (3) 12-cell reuse. (B) If a signal to interference ratio of 15 dB is required for satisfactory forward channel performance of a cellular system, what is the frequency reuse factor  (D/R) and cluster size (N) that should be used for maximum capacity if the path loss exponent is (a) n = 4, (b) n = 3 ? Assume that there are 6 co-channels cells in the first tier, and all of them are at the same distance from the mobile. Use suitable approximations. (Candidate Cluster sizes could be 3, 7, 12, 13).

3. (A) A given radio spectrum (or bandwidth) can be divided into a set of disjoint or non-interfering radio channel. All such channels can be used simultaneously while maintaining an acceptable received radio signal. Channel allocation schemes can be divided into a number of different categories depending on the comparison basis. Describe FCA (Fixed Channel Allocation), DCA (Dynamic Channel Allocation)  and HCA (Hybrid Channel Allocation). (B) Describe what is the meaning of Handoffs, intra-cell handoff, inter-cell handoff? In order to determine handoff initiations, several strategies have been proposed: Relative signal strength, Relative signal strength with threshold, Relative signal strength with hysteresis (plus optionally dwell timer). Explain the approaches.